# Features

- Silent Switcher®2 Architecture

Ultralow EMI

Eliminates PCB Layout Sensitivity

Internal Bypass Caps Reduce Radiated EMI

- SVIN: 2.7V to 5V; PVIN: 1.5V to 5V

- VOUT: 0.4V to 0.9VIN 	(LT7153SPAV-0.4);  0.5V to 0.9VIN (LT7153SPAV)

- Accurate Reference:

- 0.4V ±1.0% Over Temp (LT7153SPAV-0.4)

- 0.5V ±0.8% Over Temp (LT7153SPAV)

- Integrated N-MOSFETs: 1.8mΩ/0.7mΩ

- 15ns Min On-Time: Low Duty Cycle Operation

- Differential VOUT Remote Sense

- Configurable Fixed Output Options

- Programmable Load Line (3 settings)

- Programmable Current Limit (3 settings)

- Programmable, Synchronizable: 400kHz to 5MHz

- User Selectable Discontinuous Mode® (DCM) or Forced Continuous Mode (FCM) Operation

- Controlled TON, Current Mode: Excellent Transient

- Power Good status, Output Tracking, Clock-Out

- Supports Spread Spectrum Operation

- PolyPhase Operation: 2, 3, 4, 6, 8 or 12 Phases

- 36-Lead (4mm x 7mm) Exposed Back LQFN Package

# Applications

- Optical Module Applications

- Distributed Power Systems, Server Power

- POINT-OF-LOAD SUPPLY (I.E., ASIC, FPGA, DSP, µP)

Description

The LT®7153SP is a high efficiency monolithic synchronous, Silent Switcher®2 buck regulator capable of delivering ±25A to the load. The unique constant frequency, controlled on-time, current mode architecture is ideal for high step-down ratio applications that operate at high frequencies while demanding fast transient response. PolyPhase operation allows multiple LT7153SP regulators to run out-of-phase, which reduces the amount of input and output capacitors needed while enables paralleling for higher output currents. The operating supply voltage range is from 2.7V to 5V.

The operating frequency is programmable from 400kHz to 5MHz with an external resistor or sync to an external clock signal. The high frequency capability allows the use of physically smaller inductors and capacitors.

The LT7153SP package features the second-generation Silent Switcher architecture to minimize EMI emissions while delivering high efficiency at high switching frequencies. This includes the integration of bypass capacitors to optimize high frequency current loops and makes it easy to achieve advertised EMI performance by reducing layout sensitivity. It also features an exposed back for heat sink attachment, which can be used to significantly improve thermal performance.

|  | VREF (V) | Accuracy Over Temp |
| --- | --- | --- |
| LT7153SPAV | 0.5 | ±0.8% |
| LT7153SPAV-0.4 | 0.4 | ±1.0% |

See Figure 27 for the operation when VOUT is lower than VREF.

# Typical Application

| 0.5V, ±25A Buck Regulator |  | Efficiency and Power Loss (1MHz) |
| --- | --- | --- |

# Table of Contents

# See in PDF.

See in PDF.

# Revision History

| REVISION NUMBER | REVISION DATE | DESCRIPTION | PAGE NUMBER |
| --- | --- | --- | --- |
| 0 | 05/25 | Initial release | ─ |
| A | 07/25 | Updated figure number from Figure 31 to Figure 24 | 24 |
| B | 01/26 | Updated Figure 2 | 1 |
| C | 03/26 | Updated Features and Description sections Updated Table 1. Electrical Characteristics Updated Table 3. Pin Descriptions Updated Block Diagram Updated Figure 27, Theory of Operation and Applications Information sections Updated Ordering Guide table | 1 4, 9, 10 15 16–19 29 |

# Specifications

Electrical Characteristics

| (Specifications are at TA = 25°C. VIN = 5V, unless otherwise noted.). PARAMETER; SYMBOL; CONDITIONS/COMMENTS; MIN; TYP; MAX; UNITS. |
| --- |
| Supply Voltage Range | SVIN | -40°C ≤ TJ ≤ 125°C |  | 2.7 |  | 5 | V |
| Supply Voltage Range | PVIN |  |  | 1.5 |  | 5 | V |
| VOUT Operating Voltage | VOUT | RRT = 100kΩ | LT7153SPAV-0.4 | 0.4 |  | 0.9VIN | V |
| VOUT Operating Voltage | VOUT | RRT = 100kΩ | LT7153SPAV | 0.5 |  | 0.9VIN | V |
| Supply Operating Current2 | IQ (ACT) | Active, No-Load RRT = 100kΩ, MODE/SYNC = 0 |  |  | 1.5 | 3 | mA |
| Supply Shutdown Current2 | IQ (SHDN) | VRUN = 0V |  |  |  | 5 | µA |
| VIN OV Threshold | VIN-OV | Rising |  |  | 8.0 |  | V |
| VIN OV Threshold | VIN-OV | Falling |  |  | 7.5 |  | V |
| RUN Threshold | VRUN | Rising |  | 1.13 | 1.18 | 1.23 | V |
| RUN Threshold | VRUN_Hys | Hysteresis |  |  | 80 |  | mV |
| INTVcc Regulated Voltage | VINTVCC | SVIN > 4V |  | 3.45 | 3.6 | 3.75 | V |
| INTVcc UVLO | VUVLO | Rising |  |  | 2.55 | 2.65 | V |
| INTVcc UVLO | VUVLO | Falling |  |  | 2.4 |  | V |
| BUCK Regulator Loop |  |  |  |  |  |  |  |
| FB regulation Voltage3 | VFB | ITH = 1V | LT7153SPAV -40°C ≤ TJ ≤ 125°C | 496 | 500 | 504 | mV |
| FB Line and Load Regulation3 | ΔVFB(LINE+LOAD) | AVPSEL = 0V | -40°C ≤ TJ ≤ 125°C |  | 0.05 | 0.3 | % |
| FB Line and Load Regulation3 | ΔVFB(LINE+LOAD) | AVPSEL = Float, ITH > 1V |  |  | 1 |  | % |
| FB Line and Load Regulation3 | ΔVFB(LINE+LOAD) | AVPSEL = INTVcc, ITH > 1V |  |  | 1.5 |  | % |
| FB Pin Input Current | IFB | -40°C ≤ TJ ≤ 125°C |  | -50 |  | 50 | nA |
| Error Amplifier gm | GM(EA) | ITH = 1V |  | 0.85 | 1 | 1.15 | mS |
| Minimum On-Time | tON(MIN) | -40°C ≤ TJ ≤ 125°C |  |  | 15 | 25 | ns |
| Minimum Off-Time | tOFF(MIN) |  |  |  | 30 |  | ns |
| Positive Inductor Valley Current Limit4 | ILIM-POS |  |  | 20 | 25 | 30 | A |
| Current Threshold vs. ITH Voltage | ILIM-ITH | ITH = 1.3V |  |  | 25 |  | A |
| Current Threshold vs. ITH Voltage | ILIM-ITH | ITH = 1.0V |  |  | 0 |  | A |
| Current Threshold vs. ITH Voltage | ILIM-ITH | ITH = 0.7V |  |  | -25 |  | A |
| Negative Inductor Valley Current Limit | ILIM-NEG |  |  |  | -60 |  | A |
| Top Power NMOS On Resistance | RON-TOP | INTVcc = 3.6V |  |  | 1.8 |  | mΩ |
| Bottom Power NMOS On Resistance | RON-BOT | INTVcc = 3.6V |  |  | 0.7 |  | mΩ |
| Top Switch Leakage | ISW-TOP | VIN = 5V, VSW = 0V |  |  | 0.5 |  | µA |
| Bottom Switch Leakage | IBOT-TOP | VIN = 5V, VSW = 5V |  |  | 1 |  | µA |
| Oscillator |  |  |  |  |  |  |  |
| Oscillator Frequency | fosc | RRT = 232kΩ |  |  | 0.4 |  | MHz |
| Oscillator Frequency | fosc | RRT = 100kΩ |  | 0.85 | 1 | 1.15 | MHz |
| Oscillator Frequency | fosc | RRT = 20kΩ |  |  | 5 |  | MHz |
| SYNC Input Range | fSYNC | % of programmed frequency |  |  | ±30 |  | % |
| MODE, SYNC/SPREAD Threshold | VIL(MODE) VIL(SYNC/SPREAD) | MODE or SYNC/SPREAD LOW |  |  |  | 0.3 | V |
| MODE, SYNC/SPREAD Threshold | VIH(MODE) VIH(SYNC/SPREAD) | MODE or SYNC/SPREAD HIGH |  | 1.0 |  |  | V |
| MODE, SYNC/SPREAD Currents | IMODE ISYNC/SPREAD | MODE = 0, SYNC = 0V |  |  | 6 | 14 | µA |
| CLKOUT Threshold | VIL(CLKOUT) | CLKOUT High |  | VINTVCC – 0.2 | VINTVCC |  | V |
| CLKOUT Threshold | VIH(CLKOUT) | CLKOUT Low |  |  | 0 | 0.2 | V |
| Phasing |  |  |  |  |  |  |  |
| SYNC to Oscillator | ɸ(SYNC to OSC) | PHSMD1 = 0, PHSMD2 = 0 |  |  | 0 |  | Degree |
| SYNC to Oscillator | ɸ(SYNC to OSC) | PHSMD1 = INTVcc, PHSMD2 = 0 |  |  | 0 |  | Degree |
| SYNC to Oscillator | ɸ(SYNC to OSC) | PHSMD1 = 0, PHSMD2 = INTVcc/2 |  |  | -30 |  | Degree |
| SYNC to Oscillator | ɸ(SYNC to OSC) | PHSMD1 = INTVcc, PHSMD2 = INTVcc/2 |  |  | -45 |  | Degree |
| SYNC to Oscillator | ɸ(SYNC to OSC) | PHSMD1 = 0, PHSMD2 = INTVcc |  |  | -60 |  | Degree |
| SYNC to Oscillator | ɸ(SYNC to OSC) | PHSMD1 = INTVcc, PHSMD2 = INTVcc |  |  | -90 |  | Degree |
| Oscillator to CLKOUT | ɸ(OSC to CLKOUT) | PHSMD1 = 0, PHSMD2 = 0 |  |  | 180 |  | Degree |
| Oscillator to CLKOUT | ɸ(OSC to CLKOUT) | PHSMD1 = INTVcc, PHSMD2 = 0 |  |  | 120 |  | Degree |
| Oscillator to CLKOUT | ɸ(OSC to CLKOUT) | PHSMD1 = 0, PHSMD2 = INTVcc/2 |  |  | 180 |  | Degree |
| Oscillator to CLKOUT | ɸ(OSC to CLKOUT) | PHSMD1 = INTVcc, PHSMD2 = INTVcc/2 |  |  | 180 |  | Degree |
| Oscillator to CLKOUT | ɸ(OSC to CLKOUT) | PHSMD1 = 0, PHSMD2 = INTVcc |  |  | 120 |  | Degree |
| Oscillator to CLKOUT | ɸ(OSC to CLKOUT) | PHSMD1 = INTVcc, PHSMD2 = INTVcc |  |  | 180 |  | Degree |
| AVPSEL, PHSMD1, PHSMD2, Threshold | VIH | AVPSEL, PHSMD1, PHSMD2 High |  | VINTVCC – 0.2 |  |  | V |
| AVPSEL, PHSMD1, PHSMD2 Threshold | VIL | AVPSEL, PHSMD1, PHSMD2 Low |  |  |  | 0.2 | V |
| ILIM |  |  |  |  |  |  |  |
| Reduced Max Current Limit | ILIM55 | PHSMD1 = INTVcc /2, PHSMD2 = 0 |  |  | 14 |  | A |
| Reduced Max Current Limit | ILIM70 | PHSMD1 = INTVcc /2, PHSMD2 = INTVcc |  |  | 18 |  | A |
| Power Good |  |  |  |  |  |  |  |
| PGOOD Over Voltage Threshold | OV | VFB Rising |  | 4 | 6.5 | 9 | % |
| PGOOD Over Voltage Threshold | OV | VFB Hysteresis (Falling) |  |  | 1.5 |  | % |
| PGOOD Under Voltage Threshold | UV | VFB Falling |  | -9 | -6.5 | -4 | % |
| PGOOD Under Voltage Threshold | UV | VFB Hysteresis (Rising) |  |  | 1.5 |  | % |
| PGOOD Leakage Current | IPGOOD |  |  |  |  | 0.5 | µA |
| PGOOD Pull-Down Resistance | RPGOOD | VPG = 0.1V |  |  | 5 |  | Ω |
| PGOOD Delay | PG(Delay) | PGOOD Low to High |  |  | 6 |  | Cycles |
| PGOOD Delay | PG(Delay) | PGOOD High to Low |  |  | 25 |  | Cycles |
| Track/Soft-Start |  |  |  |  |  |  |  |
| Source Current | ITRACK/SS | VTRACK/SS = 0V |  |  | 10 | 15 | µA |
| Pull-Down Resistance | RTRACK/SS | VTRACK/SS = 0.1V |  |  | 2500 |  | Ω |

.

| 1 | The LT7153SP is tested under pulsed load conditions such that TJ ≈ TA. Specifications over the –40°C to 125°C operating junction temperature range are assured by design, characterization, and correlation with statistical process controls. The LT7153SP is guaranteed over the full –40°C to 125°C operating junction temperature range. Note that the maximum ambient temperature consistent with these specifications is determined by specific operating conditions in conjunction with board layout, the rated package thermal impedance, and other environmental factors. The junction temperature (TJ, in °C) is calculated from the ambient temperature (TA, in °C) and power dissipation (PD, in watts) according to the formula: TJ = TA + (PD • θJA), where θJA (in °C/W) is the package thermal impedance. |
| --- | --- |
| 2 | The quiescent current in Discontinuous Mode does not include switching loss of the power FETs. |
| 3 | VFB is measured in a feedback loop that servos VITH to a specified voltage. |
| 4 | Inductor should be selected such that peak inductor current does not exceed 39A in application. |

# Absolute Maximum Ratings

Absolute Maximum Ratings

| PARAMETER | RATING |
| --- | --- |
| SVIN, PVIN, SW | -0.3V to 5.5V |
| RUN, PGOOD, TRACK | -0.3V to 5.5V |
| MODE, SYNC/SPREAD | -0.3V to INTVcc + 0.3V |
| FB, ITH, RT | -0.3V to INTVcc + 0.3V |
| PHSMD1/ISEL, PHSMD2/ILIM, CLKOUT, AVPSEL | -0.3V to INTVcc + 0.3V |
| VOUT- | -0.3V to 0.3V |
| Operating Junction Temperature Range | -40°C to 125°C |
| Storage Temperature Range | -65°C to 150°C |
| Maximum Internal Temperature | 125°C |
| Peak Reflow Solder Body temperature | 260°C |

.

Stresses beyond those listed under Absolute Maximum Ratings may cause permanent damage to the device. These are stress ratings only, functional operation of the product at these or any other conditions above those indicated in the operational section of this specification is not implied. Operation beyond the maximum operating conditions for extended periods may affect product reliability.

# Pin Configurations and Function Descriptions

LQFN PACKAGE 36-LEAD (4mm x 7mm x 1.02mm) TJMAX = 125 °C, θJA = 15.8°C/W (BASED ON DEMOBOARD), θJCtop = 0.4°C/W, θJCbot = 3.4°C/W EXPOSED PAD (PIN 37) AND CORNER PINS (38~41) ARE GND, MUST BE SOLDERED TO PCB - Pin Configurations

Pin Descriptions

| PIN | NAME | DESCRIPTION |
| --- | --- | --- |
| 1 | INTVcc | Internal 3.6V regulator output. The internal power drivers and control circuits are powered from this voltage. Decouple this pin to power ground with a minimum of 4.7µF low ESR ceramic capacitor. |
| 2-12 | SW | Switch node connection of external inductor. Voltage swing of SW is from a diode voltage drop below ground to a diode voltage above PVIN. |
| 13 | RUN | Logic controlled RUN input. Do not leave this pin floating. Logic high activates the step-down regulator. |
| 14 | AVPSEL | Use this pin to select between three different load lines; tied to GND: 0.2%, float: 1%, INTVcc: 1.5%. |
| 15 | PHSMD1/ ISEL | Control inputs to the phase selector and ILIM option. A high or low state on this pin, in conjunction with the state of the PHSMD2/ILIM pin, determines the phase relationship between SYNC and the internal oscillator and between the internal oscillator and CLKOUT. When this pin is floated to mid rail (1/2 INTVcc), the PHSMD2/ILIM pin can be used to select between three lower ILIM settings. |
| 16 | PHSMD2/ ILIM | Control inputs to the phase selector and ILIM set. When PHSMD1/ISEL is high or low, this pin in conjunction with PHSMD1/ILIM determines the phasing between SYNC and the internal oscillator and between the internal oscillator and CLKOUT. When PHSMD1/ISEL is floated to mid rail (1/2 INTVcc), this pin can be used to select between three different lower ILIM settings. |
| 17 | MODE | Mode select pin. Tie MODE to GND for discontinuous mode of operation. Float or tie MODE to a voltage above 1V selects forced continuous mode. |
| 18 | PGOOD | Output power good with open-drain logic. PGOOD is pulled to ground when the voltage of the FB pin is not within ±6.5% of the internal 0.4V* reference. *0.4V: LT7153SPAV-0.4; 0.5V: LT7153SPAV |
| 19-27 | PVIN | Power VIN. Input voltage to the on-chip power MOSFETs. |
| 28 | SYNC/ SPREAD | Spread spectrum select and oscillator synchronization pin. Tie SYNC/SPREAD to GND to disable spread spectrum operation; tie to INTVcc or float to start spread spectrum. Connect to an external clock to synchronize the system clock to the external clock and put the part in forced continuous mode. |
| 29 | SVIN | Signal VIN. Filtered input voltage to the on-chip 3.6V regulator. Bypass the signal into the SVIN pin through a 2.2Ω resistor in series with 10µF ceramic capacitor. |
| 30 | CLKOUT | Output clock signal for PolyPhase operation. The phase of CLKOUT with respect to SYNC is determined by the state of PHSMD1 and PHSMD2 pins. CLKOUT’s peak-to-peak amplitude is INTVcc to GND. Minimize the parasitic capacitance on this pin to reduce phase errors. When RUN is low, this pin defaults high. |
| 31 | SGND | Ground for signal ground. |
| 32 | RT | Switching frequency programming pin. Connect an external resistor from this pin to GND to program the frequency from 400kHz to 5MHz. |
| 33 | ITH | Error amplifier output and switching regulator compensation point. The current comparator’s trip threshold is linearly proportional to this voltage, whose normal range is from 0.3V to 1.3V. |
| 34 | TRACK/SS | Output tracking and soft-start pin. Allows to control the rise time of the output voltage. Connecting a voltage between 0V and 0.4V* on this pin relative to VOUT- bypasses the internal reference input to the error amplifier. Instead, it servos the FB pin relative to VOUT- to this voltage. There’s an internal 10µA pullup current from INTVcc to this pin; so putting a capacitor from this pin to VOUT- provides a soft-start function. *0.4V: LT7153SPAV-0.4; 0.5V: LT7153SPAV |
| 35 | FB | Feedback input to the error amplifier of the step-down regulator. Connect the feedback resistor divider center tap to this pin. The output can be adjusted from 0.4V* to 4.5V. *0.4V: LT7153SPAV-0.4; 0.5V: LT7153SPAV |
| 36 | VOUT- | Negative return of output rail. Connect this pin directly to the bottom terminal of the remote output capacitor near the load in order to minimize error incurred by voltage drops across the metal traces of the PCB. |
| 37 | GND | Exposed Pad. |
| 38-41 | GND | Corner Ground Pins. |

# Typical Performance Characteristics

TA = 25°C, VIN = 3.3V, VOUT = 0.5V, unless otherwise specified.

| Efficiency vs. Load Current (500kHz) | Efficiency vs. Load Current (1MHz) |
| --- | --- |
| Efficiency vs. Load Current (2MHz) | Load Regulation vs. Load Current |
| Discontinuous Mode (DCM) Operation VIN = 3.3V, VOUT = 0.5V, IOUT = 0A, fSW = 1MHz L = 70nH, COUT = 5x100μF, RITH = 2kΩ, CITH = 10nF | Forced Continuous Mode (FCM) Operation VIN = 3.3V, VOUT = 0.5V, IOUT = 0A, fSW = 1MHz L = 70nH, COUT = 5x100μF, RITH = 2kΩ, CITH = 10nF |
| Transient Response, DCM VIN = 3.3V, VOUT = 0.5V, IOUT = 50mA to 8A, fSW = 1MHz L = 70nH, COUT = 5x100μF, RITH = 2kΩ, CITH = 10nF | Transient Response, FCM VIN = 3.3V, VOUT = 0.5V, IOUT = 50mA to 8A, fSW = 1MHz L = 70nH, COUT = 5x100μF, RITH = 2kΩ, CITH = 10nF |
| Transient Response, FCM VIN = 3.3V, VOUT = 0.5V, IOUT = 8A to 16A, fSW = 1MHz L = 70nH, COUT = 5x100μF, RITH = 1kΩ, CITH = 10nF | Output Tracking VIN = 3.3V, VOUT = 0.5V, ROUT = 0.1Ω, fSW = 1MHz L = 70nH, COUT = 5x100μF, RITH = 2kΩ, CITH = 10nF |
| Start-up Waveform VIN = 3.3V, VOUT = 0.5V, ROUT = 0.1Ω, fSW = 1MHz L = 70nH, COUT = 5x100μF, RITH = 2kΩ, CITH = 10nF CTRACK/SS = 4.7μF | Start-up Waveform (with Prebiased Output) VIN = 3.3V, VOUT = 0.5V, ROUT = 0.1Ω, fSW = 1MHz L = 70nH, COUT = 5x100μF, RITH = 2kΩ, CITH = 10nF CTRACK/SS = 4.7μF |
| Switching Frequency vs. RT | Switching Frequency vs. VOUT |
| SVIN Current vs. Switching Frequency, CCM | Switching Frequency vs. Temperature |
| Regulated FB Voltage vs. Temperature | RUN Rising Threshold vs. Temperature |
| Valley Current Limit vs. VITH | Valley Current Limit vs. Temperature |
| Case Temperature vs. Load | RDS(ON) vs. Temperature |

# Block Diagram

Block Diagram

# Theory of Operation

## Main Control Loop

The LT7153SP is a single channel, current-mode monolithic step-down regulator capable of providing ±25A of output current. In normal operation, the internal top power MOSFET is turned on for a fixed interval determined by a one-shot timer (ON signal in Block Diagram Figure 26). When the top power MOSFET turns off, the bottom power MOSFET turns on until the current comparator, ICMP, trips, restarting the one-shot timer and initiating the next cycle. Inductor current is determined by sensing the voltage drop across SW and GND nodes of the bottom power MOSFET when it is on. The voltage on the ITH pin sets the comparator threshold corresponding to the inductor valley current. The error amplifier, EA, adjusts this ITH voltage by comparing the feedback signal, VFB, with an internal 0.4V* reference. If the load current increases, it causes a drop in the feedback voltage relative to the internal reference, the ITH voltage then rises until the average inductor current matches that of the load current.

At light load currents, the inductor current can drop to zero and become negative. In Burst Mode operation, this is detected by the current reversal comparator, IREV, which then shuts off the bottom power MOSFET. Both power MOSFETs remain off with the output capacitor supplying the load current until the ITH voltage rises above zero current level to initiate the next cycle. If continuous mode of operation is desired, simply float the MODE pin or tie it to INTVcc.

The operating frequency is determined by the value of the RRT resistor, which programs the current for the internal oscillator. An internal phase-lock loop servos the switching regulator on-time to track the internal oscillator and force a constant switching frequency. A clock signal can be applied to the SYNC/SPREAD pin to synchronize the switching frequency to an external clock. The regulator defaults to force continuous operation once the clock signal is present.

The S in LT7153SP refers to the second-generation Silent Switcher technology. The IC has integrated ceramic capacitors for VIN, INTVcc, and BOOST to keep all the fast AC current loops small, thus improving the EMI performance. Furthermore, it allows for faster switching edges, which greatly improves efficiency at high switching frequencies.

## Low IQ Shutdown (RUN)

Pulling the RUN pin to ground forces the LT7153SP into its shutdown state, drawing very little current. Bringing RUN above 0.6V turns on the internal reference while keeping the MOSFETs off. Further increasing RUN above the RUN rising threshold of 1.2V nominal, the entire chip turns on. The accurate 1.2V RUN threshold allows programming the SVIN under voltage lockout threshold externally using a resistor divider.

## INTVCC Regulator and Bypass Capacitor

An internal low dropout regulator produces the 3.6V supply that powers the drivers and internal bias circuitry. A 10μF ceramic capacitor should be used to bypass INTVcc to ground; 4.7μF is the minimum value recommended. Good bypassing is necessary to supply the high transient currents required by the MOSFET drivers.

For applications running high frequency and high output currents, increased switching currents may result in higher die temperatures due to higher power dissipation across the LDO. In such cases, consider shorting INTVcc to SVIN. SVIN can be a 3.3V or 5V rail, but be aware that 5V operation results in higher switching losses, which will be more critical at higher switching frequencies. PVIN and SVIN do not have to be tied to the same potential, so an external 3.3V supply can be used to power SVIN if one is available. When biasing SVIN and PVIN from different potentials, the two supplies are required to be sequenced such that SVIN is up before PVIN powers up and that PVIN is powered down first.

## Adjustable Current Limit

The current limit setting of the LT7153SP can be lowered to 70% or 55% of its full value for applications that do not require the full 25A capability. The state of the two-phase mode pins (PHSMD1/ISEL, PHSMD2/ILIM) determines the current limit setting. See Table 1 for details.

## Overcurrent and Short-Circuit Protection

The LT7153SP has a cycle-by-cycle overcurrent protection scheme by sensing the inductor valley current. When the current limit is reached, the output begins to fall, decreasing the on-time of the top power MOSFET. If the short is prolonged enough for the on-time to reach its minimum, the off-time lengthens, lowering the switching frequency

and preventing excess current from being drawn from VIN. After the overcurrent or short is removed, the regulator

executes its soft-start function to prevent the output voltage from overshooting.

## Multiphase Operation

For output loads that demand more than 25A of current, multiple the LT7153SP outputs can be tied together to run out of phase to provide more output current. See Table 1 and Figure 28 for more information. The LT7153SP not only varies the phasing between the internal oscillator and CLKOUT, but it also varies the phasing between the external clock and the internal oscillator, allowing shorted signal chains to minimize accumulating clock jitter.

## Power Good Status Output

PGOOD open-drain output is pulled low if the regulator output feedback voltage, VFB, exits a ±6.5% window around the regulation point while the overvoltage (OV) or undervoltage (UV) comparator is tripped. This condition is released once regulation within a ±6.5% window is achieved.

Continuous operation is forced during OV and UV conditions, except during start-up when the TRACK/SS pin is ramping up to 0.4V*.

## VIN Overvoltage Protection

To protect the internal power MOSFET devices against transient voltage spikes, the LT7153SP constantly monitors the PVIN pin for an overvoltage condition. When the PVIN rises above 8V, the regulator suspends operation by shutting off both power MOSFETs. Once PVIN drops below 7.5V, the regulator immediately resumes normal operation. During an overvoltage event, the internal soft-start voltage is clamped to a voltage slightly higher than the feedback voltage. Thus, the soft-start feature is present upon exiting an overvoltage condition.

## Selectable Load Line (AVPSEL)

In order to minimize the amount of output capacitance needed, the LT7153SP allows to pick two different load line settings, in addition to a flat load line setting. Load line settings are accessed via the AVPSEL pin state: flat load line setting when this pin is tied to ground, 1% when floated, and 1.5% when tied to INTVcc.

## MODE, SYNC, Spread-Spectrum Operation

Connecting the MODE pin to ground enables Burst Mode operation for superior efficiency at low load currents at the expense of slightly higher output voltage ripple. When the MODE pin is tied to INTVcc or floated, forced continuous mode operation is selected, creating the lowest fixed output ripple at the expense of light load efficiency.

The SYNC/SPREAD pin is a dual-function pin. This pin allows the LT7153SP to sync to an external clock signal and also to activate or deactivate its spread spectrum feature. The LT7153SP detects the presence of the external clock signal on the SYNC/SPREAD pin and synchronizes the internal oscillator to the frequency of the incoming clock. Phasing between the SYNC input and the internal oscillator is determined by the states of the two phase mode pins (PHSMD1/ISEL, PHSMD2/ILIM). When the SYNC/SPREAD pin is tied to INTVcc or floated, spread-spectrum operation is enabled. The LT7153SP varies its frequency between the preset frequency and 20% higher than the preset value. When the SYNC/SPREAD pin is tied to ground, frequency spreading is disabled.

# Applications Information

A simplified LT7153SP application circuit is shown on the first page of the data sheet. External component selection is largely driven by the target current ripple, load requirement, and switching frequency. Component selection typically begins with the selection of the inductor L and resistor RRT. Once the inductor is chosen, select the input capacitor, CIN, and the output capacitor, COUT. Next, select the feedback resistors to set the desired output voltage. Finally, select the remaining optional external components for functions such as external loop compensation, tracking/soft-start, input UVLO, and PGOOD.

## Silent Switcher Architecture

The LT7153SP has integrated capacitors that allow it to operate at high switching frequencies efficiently. The internal VIN bypass capacitors allow the SW edges to transition extremely fast, effectively reducing transition loss. The capacitors also greatly reduce SW overshoot during top FET turn-on, which improves the robustness of the device over time.

## Programming Switching Frequency

Selection of the switching frequency is a trade-off between efficiency and component size. High frequency operation allows the use of smaller inductors and capacitors. Operating at lower frequencies improves efficiency by reducing internal gate charge losses but requires larger inductances and capacitances to maintain low output ripple voltage.

Connecting a resistor, RRT, from the RT pin to SGND programs the switching frequency, fSW, from 400kHz to 5MHz according to the following formula:

fSWHz=1e11RRTΩ

The internal PLL has a synchronization range of ±30% around its programmed frequency. Therefore, during external clock synchronization be sure that the external clock frequency is within this ±30% range of the RRT programmed frequency.

## Output Voltage Programming

Each regulator’s output voltage is set by an external resistive divider according to the following equation:

VOUT=0.4V*•1+R1R2

The desired output voltage is set by the appropriate selection of resistors R1 and R2, which allow the VFB pin to sense a fraction of the output voltage, as shown in Figure 27. Choosing large values for R1 and R2 results in improved zero/light load efficiency but may lead to undesirable noise coupling or phase margin reduction due to stray capacitances at the VFB node. Take care to route the VFB trace away from any noise source, such as SW trace. A feedforward compensation capacitor, CFF, can also be placed between VOUT and FB to improve transient performance.

Setting the Output Voltage

If the output voltage is outside the VON sense range (0.4V* ~ 4V), the output voltage stays in regulation, but the switching frequency may deviate from the programmed frequency. Figure 27 also illustrates how an external DAC may be used to implement dynamic output voltage control. By sourcing or sinking current at the FB node, the converter regulates VOUT accordingly, allowing VOUT to be programmed to values lower than 0.4V*.

## Soft-Start and Output Voltage TRACK

An internal 10µA pulls up the TRACK pin to INTVcc. Putting an external capacitor, CSS, from TRACK to ground enables soft starting the output to prevent a current surge on the input supply. The relationship between output rise time, TSS, and soft-start capacitance, CSS, is given by:

TSS=0.4*e5•CSS

Upon start-up time, the LT7153SP operates in discontinuous mode until track voltage is higher than 0.4V*. The regulator then operates in forced continuous mode until output is above the UV threshold. Once the output reaches this voltage, the operating mode of the regulator switches to the mode selected by the MODE pin as described above.

The LT7153SP allows the user to program its output voltage ramp rate through the TRACK pin. From 0V to 0.4V*, the TRACK voltage overrides the internal 0.4V* reference input to the error amplifier, thus regulating the feedback voltage to that of the TRACK pin. When TRACK is above 0.4V*, tracking is disabled, and the feedback voltage regulates to the internal reference voltage.

* 0.4V: LT7153SPAV-0.4

0.5V: LT7153SPAV

## Inductor Selection

For a given input voltage, VIN, output voltage, VOUT, the inductor value, L, and operating frequency, f, determine the ripple current:

∆IL=VOUTfSW•L•1-VOUTVIN

The inductor current ripple decreases with higher inductor value and higher operating frequency. Lower ripple current reduces core losses in the inductor, ESR losses in the output capacitors, and output voltage ripple. Highest efficiency operation is obtained at low frequency with small ripple current. However, achieving this requires a large inductor. There is a trade-off between component size, efficiency, and operating frequency.

A reasonable starting point is to choose a ripple current that is about 40% of IOUT(MAX). Exceeding 60% of IOUT(MAX) is not recommended. Note that the largest ripple current occurs at the highest VIN. To guarantee that ripple current does not exceed a specified maximum, the inductance should be chosen according to:

L=VOUTfSW•∆L(MAX)•1-VOUTVINMAX

Once the value for L is known, the type of inductor must be selected. Actual core loss is independent of core size for a fixed inductor value, but is very dependent on the inductance selected. As the inductance or frequency increases, core losses decrease. Unfortunately, increased inductance requires more turns of wire leading to increased DCR and copper losses.

Ferrite designs have very low core losses and are preferred at high switching frequencies, so design goals can concentrate on copper loss and preventing saturation. Ferrite core material saturates hard, which means that inductance collapses abruptly when the peak design current is exceeded. This results in an abrupt increase in inductor ripple current and consequent output voltage ripple. Do not allow the core to saturate.

To avoid overheating and poor efficiency, choose an inductor with an RMS current rating that is greater than the maximum expected output load of the application. In addition, the saturation current (typically labeled ISAT) rating of the inductor must be higher than the load current plus 1/2 of inductor ripple current:

ILPEAK=IOUTMAX+∆IL2

Different core materials and shapes change the size/current and price/current relationship of an inductor. Toroid or shielded pot cores in ferrite or permalloy materials are small and do not radiate much energy, but generally cost more than powdered iron core inductors with similar characteristics. The choice of which style inductor to use mainly depends on the price versus size requirements and any radiated field/EMI requirements.

## Input Capacitor (CIN) Selection

The input capacitance, CIN, is needed to filter the square wave current at the drain of the top power MOSFET. To prevent large voltage transients from occurring, use a low ESR input capacitor sized for the maximum RMS current. The maximum RMS current is given by:

IRMS≅IOUTMAX•VOUTVIN•VINVOUT-1

This formula has a maximum at VIN = 2VOUT, where:

IRMS≅IOUT2

This simple worst-case condition is commonly used for design because even significant deviations do not offer much relief. Note that ripple current ratings from capacitor manufacturers are often based on only 2000 hours of life, which makes it advisable to further derate the capacitor, or choose a capacitor rated at a higher temperature than required. Several capacitors may also be paralleled to meet size or height requirements in the design. For low input voltage applications, sufficient bulk input capacitance is needed to minimize transient effects during output load changes.

## Output Capacitor (COUT) Selection

The selection of COUT is determined by the effective series resistance (ESR) that is required to minimize voltage ripple and load step transients, as well as the amount of bulk capacitance that is necessary to ensure that the control loop is stable. Loop stability can be checked by viewing the load transient response. The output ripple, ∆VOUT, is determined by:

∆VOUT<∆IL•18•fSW•COUT+ESR

The output ripple is highest at maximum input voltage since ∆IL increases with input voltage. Multiple capacitors placed in parallel may be needed to meet the ESR and RMS current handling requirements.

Dry tantalum, special polymer, aluminum electrolytic, and ceramic capacitors are all available in surface mount packages. Special polymer capacitors are very low ESR but have lower capacitance density than other types. Tantalum capacitors have the highest capacitance density, but it is important to only use types that are surge tested for use in switching power supplies. Aluminum electrolytic capacitors have significantly higher ESR, but can be used in cost-sensitive applications provided that consideration is given to ripple current ratings and long-term reliability. Ceramic capacitors have excellent low ESR characteristics and small footprints.

Since the ESR of a ceramic capacitor is so low, it is more useful to choose the output capacitor value to fulfill a charge storage requirement. During a load step, the output capacitor must instantaneously supply the current to support the load until the feedback loop raises the switch current enough to support the load. The time required for the feedback loop to respond is dependent on the compensation and the output capacitor size. Typically, five cycles are required to respond to a load step, but only in the first cycle does the output voltage drop linearly. The output droop, VDROOP, is usually about three times the linear drop of the first cycle. Thus, a good place to start with the output capacitor value is approximately.

COUT=3•∆IOUTfO•VDROOP

## Using Ceramic Input and Output Capacitors

Higher values, lower cost ceramic capacitors are now becoming available in smaller case sizes. Their high ripple current, high voltage rating, and low ESR make them ideal for switching regulator applications. However, due to the self-resonant and high Q characteristics of some types of ceramic capacitors, take care when these capacitors are used at the input and output. When a ceramic capacitor is used at the input and the power is supplied by a wall adapter through long wires, a load step at the output can induce ringing at the VIN input. At best, this ringing can couple to the output and be mistaken as loop instability. At worst, a sudden inrush of current through the long wires can potentially cause a voltage spike at VIN large enough to damage the part. For a more detailed discussion, refer to Application Note 88.

When choosing the input and output ceramic capacitors, choose the X5R and X7R dielectric formulations. These dielectrics have the best temperature and voltage characteristics of all the ceramics for a given value and size. Choose X8R for 150°C applications. More capacitance may be required depending on the duty cycle and load step requirements. In most applications, the input capacitor is merely required to supply high frequency bypassing, since the impedance to the supply is very low. A 47μF ceramic capacitor is usually enough for these conditions. Place this input capacitor as close to the PVIN and GND pins as possible.

## Minimum Off-Time and Minimum On-Time Considerations

The minimum off-time, tOFF(MIN), is the smallest amount of time that the LT7153SP is capable of turning on the bottom power MOSFET, tripping the current comparator and turning the power MOSFET back off. This time is generally about 30ns. The minimum off-time limit imposes a maximum duty cycle of tON/(tON + tOFF(MIN)). If the maximum duty cycle is reached due to a dropping input voltage, for example, then the output drops out of regulation. The minimum input voltage to avoid dropout is:

VINMIN=VOUT•tON+tOFFMINtON

Conversely, the minimum on-time is the smallest duration of time in which the top power MOSFET can be in its ON state. This time is typically 15ns. In continuous mode operation, the minimum on-time limit imposes a minimum duty cycle of

DC(MIN)=fSW•tONMIN

where tON(MIN) is the minimum on-time. Reducing the operating frequency alleviates the minimum duty cycle constraint.

In the rare cases where the minimum duty cycle is surpassed, the output voltage remains in regulation and the switching frequency decreases from its programmed value. This is an acceptable result in many applications. So, this constraint may not be of critical importance in most cases. High switching frequencies may be used in the design without any fear of output overvoltage. As the sections on inductors and capacitor selection show, high switching frequencies allow the use of smaller board components, thus reducing the size of the application circuit.

## Multiphase Configuration

Polyphase operation allows multiple LT7153SP regulators to run out of phase, 2, 3, 4, 6, 8 or 12 Phases (see Figure 28), which reduces the amount of input and output capacitors needed while enables paralleling for higher output currents. By setting up PHASE1/SEL and PHSMD2/ILIM pin (see Table 4), it determines the phase relationship between the internal oscillator and CLKOUT and between the internal oscillator and SYNC.

2, 3, 4, 6, 8, 12 Multiphase Configuration

Multiphase Pin Configuration

| SYNC to Oscillator | (SYNC to OSC) | PHSMD1 = 0, PHSMD2 = 0 | 0 | Degree |
| --- | --- | --- | --- | --- |
|  |  | PHSMD1 = INTVCC, PHSMD2 = 0 | 0 |  |
|  |  | PHSMD1 = 0, PHSMD2 = INTVCC/2 | -30 |  |
|  |  | PHSMD1 = INTVCC, PHSMD2 = INTVCC/2 | -45 |  |
|  |  | PHSMD1 = 0, PHSMD2 = INTVCC | -60 |  |
|  |  | PHSMD1 = INTVCC, PHSMD2 = INTVCC | -90 |  |
| Oscillator to CLKOUT | (OSC to CLKOUT) | PHSMD1 = 0, PHSMD2 = 0 | 180 | Degree |
|  |  | PHSMD1 = INTVCC, PHSMD2 = 0 | 120 |  |
|  |  | PHSMD1 = 0, PHSMD2 = INTVCC/2 | 180 |  |
|  |  | PHSMD1 = INTVCC, PHSMD2 = INTVCC/2 | 180 |  |
|  |  | PHSMD1 = 0, PHSMD2 = INTVCC | 120 |  |
|  |  | PHSMD1 = INTVCC, PHSMD2 = INTVCC | 180 |  |

## ITH Compensation

Proper ITH components should be selected for OPTI-LOOP® optimization. The compensation network is shown in Figure 29. The RC filter sets the dominant pole-zero loop compensation. The gain of the loop increases with the RITH and the bandwidth of the loop increases with decreasing CITH. If RITH is increased by the same factor that CITH is decreased, the zero frequency is kept the same, thereby keeping the phase the same in most critical frequency ranges of the feedback loop.

External Compensation Network

For a 1MHz application, an R-C network of 4.7nF and 2kΩ provides a good starting point. A 4.7pF bypass capacitor, CITHP, on the ITH pin is recommended for the purposes of filtering out high frequency coupling from stray board capacitance. Table 5 provides a basic guideline for the compensation values to use, given the frequency of the part. Slight tweaks to those values may be required depending on the amount of output capacitance used in the application.

Compensation Values

| Frequency | RITH | CITH | CTHP |
| --- | --- | --- | --- |
| 1MHz | 2k | 4.7nF | 4.7pF |
| 2MHz | 2k | 2.2nF | 4.7pF |

## Checking Transient Response

The OPTI-LOOP compensation allows the transient response to be optimized for a wide range of loads and output capacitors. The availability of the ITH pin not only allows for optimization of the control loop behavior but also provides a DC-coupled and AC-filtered closed loop response test point. The DC step, rise time, and settling at this test point truly reflects these close loop response. Assuming a predominantly second order system, phase margin and/or damping factor can be estimated using the percentage of overshoot seen at this pin.

The ITH external component shown in the Table 5 circuit provides an adequate starting point for most applications. The RC filter sets the dominant pole-zero loop compensation. The values can be modified slightly (from 0.5 to 2 times their suggested value) to optimize transient response once the final PCB layout is done and the particular output capacitor type and value are determined.  Select the output capacitors because their various types and values determine the loop feedback factor gain and phase. In addition, add a feedforward capacitor CFF to improve the high frequency response, as shown in Figure 29. Capacitor CFF provides phase lead by creating a high frequency zero with R2, which improves the phase margin.

An output current pulse of 20% to 100% of full load current having a rise time of ~1µs produces output voltage and ITH pin waveforms that give a sense of the overall loop stability without breaking the feedback loop. Switching regulators may take several cycles to respond to a step in load current. When a load step occurs, VOUT immediately shifts by an amount equal to the ∆ILOAD  ESR, where ESR is the effective series resistance of COUT. ∆ILOAD also begins to charge or discharge COUT, generating a feedback error signal used by the regulator to return VOUT to its steady-state value. During this recovery time, VOUT can be monitored for overshoot or ringing that indicates a stability problem.

The initial output voltage step may not be within the bandwidth of the feedback loop, so the standard second order overshoot/DC ratio cannot be used to determine phase margin. The output voltage settling behavior is related to the stability of the closed-loop system and demonstrates the actual overall supply performance. For detailed explanation of optimizing the compensation components, including a review of control loop theory, refer to Analog Devices Application Note 76.

In some applications, a more severe transient can be caused by switching in loads with large (>47µF) input capacitors. The discharge input capacitors are effectively put in parallel with COUT, causing a rapid drop in VOUT. No regulator can deliver enough current to prevent this problem if the switch connecting the load has low resistance and is driven quickly. The solution is to limit the turn-on speed of the load switch driver. A hot swap controller is designed specifically for this purpose and usually incorporates current limiting, short-circuit protection, and soft-starting.

## Thermal Conditions

In some applications where the LT7153SP is operated at a combination of high ambient temperature, high switching frequency, high VIN, and high output load, the required power dissipation might push the part to exceed its maximum junction temperature. To avoid the LT7153SP from exceeding the maximum junction temperature, maximum current rating is derated depending on the operating conditions. The temperature rise of the part varies depending on the thickness of copper on the PCB board, the number of layers of the board, and the shape of copper trace. In general, a thick continuous piece of copper on the top layer of the PCB for SW and GND pins greatly improves the thermal performance of the part.

The LT7153SP offers exposed die back on the package top for heat sink mount. This option provides the capability to improve thermal performance for the same load if an appropriately sized heat sink is mounted correctly on the package. Figure 24 shows case temperature rise of the LT7153SP on a standard 6-layer, 2oz copper per layer PCB board (standard demo board).

## Efficiency Considerations

The percent efficiency of a switching regulator is equal to the output power divided by the input power times 100%. It is often useful to analyze individual losses to determine what is limiting the efficiency and which change produces the most improvement. Percent efficiency can be expressed as:

%Efficiency=100%-L1+L2+L3+…

where L1, L2, etc., are the individual losses as a percentage of input power.

Although all dissipative elements in the circuit produce losses, three main sources usually account for most of the losses in LT7153SP circuits: 1) I2R losses, 2) switching and biasing losses, 3) other losses.

- I2R losses are calculated from the DC resistances of the internal switches, RSW, and external inductor, RL. In continuous mode, the average output current flows through inductor L but is chopped between the internal top and bottom power MOSFETs. Thus, the series resistance looking into the SW pin is a function of both top and bottom MOSFET RDS(ON) and the duty cycle (DC) as follows:

RSW=RDSONTOPDC+RDSONBOT1-DC

The RDS(ON) for both the top and bottom MOSFETs can be obtained from the Typical Performance Characteristics curves. Thus to obtain I2R losses:

I2R losses=IOUT2RSW+RL

- The switching current is the sum of the MOSFET driver and control currents. The power MOSFET driver current results from switching the gate capacitance of the power MOSFETs. Each time a power MOSFET gate is switched from low to high to low again, a packet of charge dQ moves from PVIN to ground. The resulting dQ/dt is a current out of IN that is typically much larger than the DC control bias current. In continuous mode, IGATECHG = f(QT + QB), where QT and QB are the gate charges of the internal top and bottom power MOSFETs, and f is the switching frequency. The power loss is thus:

Switching Loss=IGATECHG• PVIN

The gate charge loss shows up as current through the INTVcc pin as well as frequency. Thus, their effects are more pronounced in applications with higher input voltage and higher frequency.

- Other hidden losses such as transition loss, copper trace, and internal load resistances can account for additional efficiency degradations in the overall power system. It is very important to include these system level losses in the system design. Transition loss arises from the brief amount of time the top power MOSFET spends in the saturated region during switch node transitions. The LT7153SP internal power devices switch quickly enough that these losses are not significant compared to other sources. Other losses including diode conduction losses during dead-time and inductor core losses, which generally account for less than 2% total additional loss.

## Design Example

As a design example, consider the LT7153SP in an application with the following specifications:

VIN = 3.3V to 5V

VOUT = 0.875V

IOUT(MAX) = 25A

fSW = 2MHz

First, to program the output to 0.875V, set R2 to be 10kΩ and R1 to be 7.5kΩ, per Figure 27. For best accuracy, use a 0.1% resistor.

For a typical soft-start time of 4ms (0% to 100% of final VOUT value), the CSS should be:

10μA=CSS•0.875V4ms

CSS = 46nF

Use a typical 47nF capacitor for CSS.

Because efficiency is important at both high and low load current, discontinuous mode operation is utilized. Select from the characteristic curves the correct RT resistor for the 2MHz switching frequency. Based on that, RRT should be 50kΩ. Then calculate the inductor value to achieve a current ripple that is about 40% of the maximum peak current limit (25A) at maximum VIN:

L=VOUTfSW•∆L(MAX)•1-VOUTVINMAX

L=0.875V2MHz•10A• 1-0.875V5V=36nH

The closest standard value inductor is 36nH.

Select COUT based on the ESR that is required to satisfy the output ripple requirement and the bulk capacitance needed for loop stability. For this design, use two 100µF and two 220µF ceramic capacitors.

∆VOUT<∆IL•18•fSW•COUT+ESR

Size CIN for a maximum current rating of:

IRMS≅IOUTMAX•VOUTVIN•SqrtVINVOUT-1

IRMS=25A •(0.875V/5V) •((5V/0.875V)-1)12=9.5A

Decoupling VIN with two 47µF ceramic capacitor, as shown in Figure 30, is adequate for most applications.

## Board Layout Considerations

When laying out the printed circuit board, use the following checklist to ensure proper operation of the LT7153SP (see Figure 30). Check the following in the layout:

- Place the capacitors CIN connect to the PVIN and GND as close as possible. These capacitors provide the AC current to the internal power MOSFETs and their drivers.

- Are COUT and L closely connected? The (–) plate of COUT returns current to GND and the (–) plate of CIN.

- Place the FB dividers close to the part with kelvin connections to VOUT and GND at the point of load.

- Keep sensitive components away from the SW pin. Route the FB resistors, RT resistor, the compensation component, and the INTVcc bypass caps away from the SW trace and the inductor.

- A ground plane is preferred.

- Flood all unused areas on all layers with copper, which reduces the temperature rise of power components. Connect these copper areas to GND.

Example of Top Layer PCB Design

# Typical Applications

0.875V/25A, 2MHz, Step-Down Converter (with 1% Load Line)

High Efficiency, Dual Phase 0.5V/50A, 2MHz, Step-Down Converter

# Outline Dimensions

36-Terminal Land Grid Array [LGA]

(7mm x 4mm x 1.02mm)

(Reference DWG # CC-36-3)

# Ordering Guide

| TAPE AND REEL (MINI) | TAPE AND REEL | PART MARKING | FINISH CODE | PAD FINISH | PACKAGE TYPE | MSL RATING | TEMPERATURE RANGE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LT7153SPAV#TRMPBF | LT7153SPAV#TRPBF | 7153SPV | E4 | Au (RoHS) | LQFN (Laminate Package with QFN Footprint) | 3 | -40°C to 125°C |
| LT7153SPAV-0.4#TRMPBF | LT7153SPAV-0.4#TRPBF | 153SP04 | E4 | Au (RoHS) | LQFN (Laminate Package with QFN Footprint) | 3 | -40°C to 125°C |

- Parts ending with PBF are RoHS and WEEE compliant.

- Pad and ball finish code is per IPC/JEDEC J-STD-609.

- Temperature grades are identified by a label on the shipping container. Contact the factory for parts specified with wider operating temperature ranges.

- TRM = 500 pieces.

- Recommended PCB Assembly and Manufacturing Procedures.

- Package and Tray Drawings.

# Related Parts

| PART NUMBER | DESCRIPTION | COMMENTS |
| --- | --- | --- |
| LT7200S | 18V, Quad ±5A/Quad Phase ±20A Silent Switcher®2 Step-Down Regulator | 2.9V < VIN < 18V, 0.5V < VOUT < 0.9VIN, 96% Maximum Efficiency, 5mm x 6mm LQFN-48 Package |
| LTC7150S | 20V, 20A Synchronous Step-Down Regulator | 3.1V < VIN < 20V, 0.6V < VOUT < 5.5V, 96% Maximum Efficiency, 5mm × 6mm BGA Package |
| LTC3605/ LTC3605A | 20V, 5A Synchronous Step-Down Regulator | 4V < VIN < 20V, 0.6V < VOUT < 20V, 96% Maximum Efficiency, 4mm × 4mm QFN-24 Package |
| LTC3613 | 24V, 15A Monolithic Step-Down Regulator with Differential Output Sensing | 4.5V < VIN < 24V, 0.6V < VOUT < 5.5V, 0.67% Output Voltage Accuracy, Valley Current Mode, Programmable from 200kHz to 1MHz, Current Sensing, 7mm × 9mm QFN-56 Package |
| LTC3622 | 17V, Dual 1A Synchronous Step-Down Regulator with Ultralow Quiescent Current | 2.7V < VIN < 17V, 0.6V < VOUT < VIN, 95% Maximum Efficiency, 3mm × 4mm DFN-14 and MSOP-16 Package |
| LTC3623 | 15V, ±5A Rail-to-Rail Synchronous Buck Regulator | 4V ≤ VIN ≤ 15V, 96% Maximum Efficiency, 3mm × 5mm QFN Package |
| LTC3624 | 17V, 2A Synchronous Step-Down Regulator with 3.5µA Quiescent Current | 2.7V < VIN < 17V, 0.6V < VOUT < VIN, 95% Maximum Efficiency, 3.5µA IQ, Zero-Current Shutdown, 3mm × 3mm DFN-8 Package |
| LTC3633A/ LTC3633A-1 | Dual Channel 3A, 20V Monolithic Synchronous Step-Down Regulator | 3.6V < VIN < 20V, 0.6V < VOUT < VIN, 95% Maximum Efficiency, 4mm × 5mm QFN-28 and TSSOP-28 Package |
| LTM4639 | Low VIN 20A DC/DC µModule Step-Down Regulator | Complete 20A Switch Mode Power Supply, 2.375V < VIN < 7V, 0.6V < VOUT < 5.5V, 1.5% Maximum Total DC Output Voltage Error, Differential Remote Sense Amp, 15mm × 15mm BGA Package |
| LTM4637 | 20A DC/DC µModule Step-Down Regulator | Complete 20A Switch Mode Power Supply, 4.5V < VIN < 20V, 0.6V < VOUT < 5.5V, 1.5% Maximum Total DC Output Voltage Error, Differential Remote Sense Amp, 15mm × 15mm BGA or LGA Package |
| LTC7130 | 20V, 20A Monolithic Buck Converter with Ultralow DCR Sensing | 4.5V < VIN < 20V, 95% Maximum Efficiency, Optimized for Low Duty Cycle Applications, 6.25mm × 7.5mm BGA Package |
| LT8642S | 18V, 10A Synchronous Step-Down Silent Switcher 2 | 2.8V < VIN < 18V, 0.6V < VOUT < VIN, 96% Maximum Efficiency, 4mm × 4mm 0.94mm LQFN Package |